Damian Cisneros

CPE301 – SPRING 2018

# Design Assignment 2

DO NOT REMOVE THIS PAGE DURING SUBMISSION:

The student understands that all required components should be submitted in complete for grading of this assignment.

| NO | SUBMISSION ITEM | COMPLETED (Y/N) | MARKS (/MAX) |
| --- | --- | --- | --- |
| 1 | COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS |  |  |
| 2. | INITIAL CODE OF TASK 1/A |  |  |
| 3. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 2/B |  |  |
| 3. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 3/C |  |  |
| 3. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 4/D |  |  |
| 3. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 5/E |  |  |
| 4. | SCHEMATICS |  |  |
| 5. | SCREENSHOTS OF EACH TASK OUTPUT |  |  |
| 5. | SCREENSHOT OF EACH DEMO |  |  |
| 6. | VIDEO LINKS OF EACH DEMO |  |  |
| 7. | GOOGLECODE LINK OF THE DA |  |  |

Damian Cisneros

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

List of Components used:

ATmega328p

Breadboard

1k resistor

Pushbutton for switch

LED

Block diagram with pins used in the Atmega328P

- INITIAL/DEVELOPED CODE OF TASK 1/Assembly

;

; DA2_Task1_asm.asm

;

; Created: 3/15/2018 2:18:30 PM

; Author : Damian Cisneros

; Description : Program generates waveform with 0.5s period and 50% duty cycle

;               waveform is output to LED connected to PINB2 (uses 8MHz clock)

;

.ORG 0

LDI R21, 0 ;Used to toggle LED

SBI DDRB, 2 ;Set PINB2 to output

LDI R20, 8

//STS TCCR1B, R20 ;Set to CTC mode

LDI R20, 13

STS TCCR1B, R20 ;Set prescalar to 1024

LDI R20, 0X0A ;Higher bits of 3905 calculated as 0.25s

LDI R21, 0X07 ;Lower bits of 3905 calculated as 0.25s

STS OCR1AH, R21 ;Set output compare register high bits

STS OCR1AL, R20 ;Set output compare register low bits

LOOP:

IN R20, TIFR1 ;Grab TIFR0 (Interrupt Flag Register)

ANDI R20, 4 ;Isolate OCFA (Output compare A match flag)

BREQ LOOP ;Check if OCFA flag is set. If it is move on to next instruction.

;Else loop back

EOR R21, R20;Toggle LED

OUT PORTB, R21 ;Output LED

LDI R20, 4 ;Used to clear TIFR flag

OUT TIFR1, R20 ; Clear TIFR flag

RJMP LOOP

- INITIAL/DEVELOPED CODE OF TASK 1/C Code

/*

* DA2_Task1.c

*

* Created: 3/15/2018 1:02:45 PM

* Author : Damian Cisneros

* Description : Program generates waveform with 0.5s period and 50% duty cycle

*               waveform is output to LED connected to PINB2 (uses 8MHz clock)

*/

#include <avr/io.h>

int main(void)

{

int maxCT=3905; //set max calculated for 0.5s

DDRB= 0b00000100; //set PINB2 to output

TCCR1A = 0; //normal mode

TCCR1B = 5; //set prescale: 1024

while (1)

{

if(TCNT1>=(maxCT/2)) //50% duty cycle

{

TCNT1=0; //reset count

PORTB ^= DDRB; //toggle led

}

}

return 0;

}

- INITIAL/DEVELOPED CODE OF TASK 2/Assembly

;

; DA2_Task2_asm.asm

;

; Created: 3/15/2018 7:10:59 PM

; Author : Damian Cisneros

; Description : This program lights up LED on PINB2 1s after a pushbutton is

;               pressed on PORTD2 (8MHz clock)

.ORG 0

CBI DDRD, 2 ;set PIND2 to input

SBI DDRB, 2 ;set PINB2 to output

LDI R20, 0XFF ;Turn on pull-up resistor for PIND2

OUT PORTD, R20

LDI R20, 13

STS TCCR1B, R20 ;Set CTC mode with prescalar 1024

LOOP:

IN R20, PIND ;grab PIND

NEG R20 ;Pull-up resistor logic

ANDI R20, 4 ;check PIND2 input

BREQ LOOP

LDI R20,0x00

STS TCNT1H,R20 ;resetting the counter to 0

LDI R20,0x00

STS TCNT1L,R20 ;resetting the counter to 0

RCALL delay ;delay 1s

LDI R20, 4

OUT PORTB, R20 ;turn on LED for 1s

RCALL delay ;delay 1s

LDI R20, 0

OUT PORTB, R20 ;turn off LED

RJMP LOOP

delay:

LDS R29, TCNT1H ;loading upper bit of counter to R29

LDS R28, TCNT1L ;loading lower bit of counter to R28

CPI R28, 0x41 ;comparing if lower is 0x41

BRSH body ;if false repeat

RJMP delay ;if true repeat

body:

CPI R29,0x0f ;comparing if higher is 0x0f

BRSH done

RJMP delay

done:

LDI R20,0x00

STS TCNT1H,R20 ;resetting the counter to 0 for next round

LDI R20,0x00

STS TCNT1L,R20 ;resetting the counter to 0 for next round

RET

- INITIAL/DEVELOPED CODE OF TASK 2/C Code

/*

* DA2_Task2_c.c

*

* Created: 3/15/2018 6:03:57 PM

* Author : Damian Cisneros

* Description: Program lights up LED in PORTB2 1s after Pushbutton is pressed

*              which stays lit for 2s before it turns off again.

*/

#define F_CPU 8000000UL //define 8MHz

#include <avr/io.h>

#include <util/delay.h>

int main(void)

{

DDRD = 4; //set PIND2 to input

DDRB = 4; //set PINB2 to output

PORTB |= 0b00000100; //Turn on pull-up resistor

while (1)

{

if(PIND & 0b00000100){ //check if PIND2 is set

_delay_ms(1000); //delay 1s

PORTB |= (PINB2 << 1); //turn on LED

_delay_ms(2000); //delay 2s

}

else{

PORTB &= ~(PINB2 << 1); //turn off LED

}

}

}

- INITIAL/DEVELOPED CODE OF TASK 3/C Code

/*

* DA2_Task3_c.c

*

* Created: 3/16/2018 10:49:48 AM

* Author : Damian Cisneros

* Description : This program works with 1MHz clock and toggles an LED in PINB2

*               every .5s with a duty cycle of 50% which toggles at .25s rate

*/

//clock is 1MHz

#include <avr/io.h>

int main(void)

{

DDRB = 4; //set PINB2 as output

TCCR1B = 5; //set CTC mode with 1024 prescale

TCNT1 = 65293; //calculated count for 0.5s/2 for 50% duty cycle

while (1)

{

if (TIFR1 & (1 << TOV1)){ //check overflow flag

PORTB ^= DDRB; //toggle LED

TCNT1 = 65293; // reload calculated count for 0.5s/2 for 50% duty cycle

TIFR1 |= (1 << TOV1); //clear overflow flag

}

}

}

- INITIAL/DEVELOPED CODE OF TASK 4/C Code

/*

* DA2_Task4_c.c

*

* Created: 3/15/2018 9:26:13 PM

* Author : Damian Cisneros

* Description : This program blinks LED connected to PINB2 with a square wave

*               at a rate 0.25s which is 50% duty cycle of 0.5s using TIMER1

*               OVF interrupt (1MHz clock)

*/

#include <avr/io.h>

#include <avr/interrupt.h>

ISR(TIMER1_OVF_vect){

PORTB ^= DDRB; //toggle LED

TCNT1 = 65293; //reset TCNT1 to 0.25s calculated based on 50% duty cycle of 0.5s

}

int main(void)

{

DDRB = 4; //set PORTB2 as output

TCCR1B = 5; //set prescalar 1024

TCNT1 = 65293; //set TCNT1 to 0.25s calculated based on 50% duty cycle of 0.5s

TIMSK1 |= (1 << TOIE1); //enable OVF interrupt

sei();  //enable global interrupts

while (1)

{

}

}

- INITIAL/DEVELOPED CODE OF TASK 5/C Code

/*

* DA2_Task5_c.c

*

* Created: 3/15/2018 9:47:24 PM

* Author : Damian Cisneros

* Description : Program lights up LED 1s after INT0 interrupt occurs by

*               pressing the button. It stays on for 2s then turns off

*               (1MHz clock, LED connected to PINB2, pushbutton INT0 pin)

*/

#include <avr/io.h>

#include <avr/interrupt.h>

#define F_CPU 1000000UL

#include <util/delay.h>

int main(void)

{

DDRB = 4; //set PORTB2 as output for LED

DDRD &= ~(1 << PIND2); //set PORTD2 as input for pushbutton switch

EIMSK = (1 << INT0); //enable INT0 interrupt

EICRA = 3; //enable INT0 flag on rising edge

sei(); //enable global interrupt

while (1)

{

}

}

ISR(INT0_vect){

_delay_ms(5); //debounce

if (PIND & 0x04){

_delay_ms(1000); //initial 1s delay after pressing button

PORTB |= (1 << PINB2); //turn on LED connected to PINB2 for 2s

_delay_ms(2000); //delay on mode for 2s

PORTB &= ~(1 << PINB2); //turn off LED

}

}

- SCHEMATICS

Task 1, 3, and 4 Assembly and C Board Schematic

Task 2 and Task 5 Assembly and C Board Schematic

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

Task 1 Assembly and C Board Setup

Task 2 Assembly and C Board Setup

Task 3 C Board Setup

Task 4 C Board Setup

Task 5 C Board Setup

- VIDEO LINKS OF EACH DEMO

Playlist - https://www.youtube.com/playlist?list=PL5RuXbzEXwes4cYuR7YLbzXgePpAIsU8P

Task 1/Assembly - https://youtu.be/_-Z28W37Cok

Task 1/C Code - https://youtu.be/O0jzXN2m8TA

Task 2/Assembly - https://youtu.be/8KltKFdTjsA

Task 2/C Code - https://youtu.be/gRL5-Tx6MFY

Task 3/C Code - https://youtu.be/vfq_NPs05nc

Task 4/C Code - https://youtu.be/jwo2JOPgxP4

Task 5/C Code - https://youtu.be/OTYh48hzvEs

- GITHUB LINK OF THIS DA

https://github.com/cisned2/DA2

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.